SimulinkSDK instruction manual

- folder

（1）、XINGYING_SimulinkDemo -------- The source code of client receiving program for reference.

（2）、lib -------- The directory of library.

- XINGYING_SimulinkDemo SDK data receiving

（1）、Set the PC IP address to 10.1.1.198 and subnet mask to 255.255.255.0. Disable the firewall and network blocking software.

（2）、Run XINGYING motion capture software as an administrator.

（3）、Select the data broadcast interface, select “NIC Address：10.1.1.198”.

（4）、check “Setting”,“SDK Enabled”.XINGYING sends motion capture data.

（5）、In live mode, click run;Or, in post-process mode, play back data.

（6）、Run XINGYING_SimulinkDemo.slx to receive SDK data.

This program receives data by default on the computer at 10.1.1.198, and the data received is the skeletal data of the i-th Segment of Segments in the i-th Markerset of Markersets. (Frame number: FrameNo; Displacement information: Tx, Ty, Tz; Rotation information: Qx, Qy, Qz, Qw)

Note: In post-process mode, you need to close the client receiver before switching the motion capture data.

iMarkerset and iSegments Start taking values from 0

- Precautions for using XINGYING_SDK in Simulink (explained using the S-Function Builder method in the demo)

XINGYING_SDK

（1）、Set the programming language to C++

（2）、Include dynamic library

nokov_sdk.lib

（3）、Include header file

#include <math.h>

#include <stdio.h>

#include <tchar.h>

#include <conio.h>

#include <winsock2.h>

#include "NokovSDKTypes.h"

#include "NokovSDKClient.h"

#include <iostream>

（4）、Function declaration

unsigned int MyServersDataPort = 3130;

unsigned int MyServersCommandPort = 3131;

NokovSDKClient* theClient;

char szMyIPAddress[128] = "";

char szServerIPAddress[128] = "";

int iMarkerset_in=0;

int iSegments_in=0;

int FrameNo_out=0;

int curFrmNo = 0;

double Tx_out=0;

double Ty_out=0;

double Tz_out=0;

double Qx_out=0;

double Qy_out=0;

double Qz_out=0;

double Qw_out=0;

Note: If multiple outputs need to be defined, intermediate variables can be added here.

（5）、Set sampling

（6）、Set the input port and output port, the data type of the input port is int32, and the data type of the output port is double

Note: If multiple inputs or outputs are needed, you can add input or output ports here.

（7）、Initialization code added in the initial part

// create NokovSDK client

theClient = new NokovSDKClient();

int retCode = -1;

retCode = theClient->Initialize("10.1.1.198");//szMyIPAddress:Client IP (local IP);szServerIP:Server IP (computer IP running motion capture software)

（8）、Code for receiving data added in the output part (if you need to output more skeletal data, you can modify or add ports accordingly, and add data output code)

iMarkerset_in = iMarkerset[0];//the i-th Markerset of the motion capture data

iSegments_in = iSegments[0];//the j-th RigidBody (RigidBody)

sFrameOfMocapData* data = theClient->GetLastFrameOfMocapData();

if (data !=NULL)

{

curFrmNo = data->iFrame;

//Case1:Create Rigid Markerset

if(iMarkerset_in>=data->nRigidBodies)

{

// If the Markerset number input is incorrect, the output is set to 0

Tx_out = 0;

Ty_out = 0;

Tz_out = 0;

Qx_out = 0;

Qy_out = 0;

Qz_out = 0;

Qw_out = 0;

}

else

{

Tx_out = (real_T) data->RigidBodies[iMarkerset_in].x;

Ty_out = (real_T) data->RigidBodies[iMarkerset_in].y;

Tz_out = (real_T) data->RigidBodies[iMarkerset_in].z;

Qx_out = (real_T) data->RigidBodies[iMarkerset_in].qx;

Qy_out = (real_T) data->RigidBodies[iMarkerset_in].qy;

Qz_out = (real_T) data->RigidBodies[iMarkerset_in].qz;

Qw_out = (real_T) data->RigidBodies[iMarkerset_in].qw;

}

//Case2:Create Body Markerset

//      if(iMarkerset[0]>=data->nSkeletons)

//      {

//          // If the Markerset number input is incorrect, the output is set to 0

//          Tx_out = 0;

//          Ty_out = 0;

//          Tz_out = 0;

//          Qx_out = 0;

//          Qy_out = 0;

//          Qz_out = 0;

//          Qw_out = 0;

//       }

//       else

//       {

//          Tx_out = data->Skeletons[iMarkerset_in].RigidBodyData[iSegments_in].x;

//          Ty_out = data->Skeletons[iMarkerset_in].RigidBodyData[iSegments_in].y;

//          Tz_out = data->Skeletons[iMarkerset_in].RigidBodyData[iSegments_in].z;

//          Qx_out = data->Skeletons[iMarkerset_in].RigidBodyData[iSegments_in].qx;

//          Qy_out = data->Skeletons[iMarkerset_in].RigidBodyData[iSegments_in].qy;

//          Qz_out = data->Skeletons[iMarkerset_in].RigidBodyData[iSegments_in].qz;

//          Qw_out = data->Skeletons[iMarkerset_in].RigidBodyData[iSegments_in].qw;

//        }

}

else

{

Sleep(5);// Reduce CPU usage

}

*FrameNo = (real_T) curFrmNo;

*Tx = (real_T) Tx_out;

*Ty = (real_T) Ty_out;

*Tz = (real_T) Tz_out;

*Qx = (real_T) Qx_out;

*Qy = (real_T) Qy_out;

*Qz = (real_T) Qz_out;

*Qw = (real_T) Qw_out;

theClient->NokovFreeFrame(data);

（9）、Add code to the end part

theClient->Uninitialize();

delete theClient

- Analysis of the data

（1）、Initialize（XINGYING software NIC Address）

int retCode = -1;

retCode = theClient->Initialize(szServerIP);//szMyIPAddress：Client IP (local IP)；szServerIP：Server IP (computer IP running motion capture software)

（2）、Get Data Descriptions

sServerDescription ServerDescription;

memset(&ServerDescription, 0, sizeof(ServerDescription));

theClient->GetServerDescription(&ServerDescription);

sDataDescriptions* ps = nullptr;

theClient->GetDataDescriptions(&ps);

dataDescription note：

ps->nDataDescriptions ------ DataDescriptions

const auto& dataDescription = ps->arrDataDescriptions[dsIndex];

dataDescription.type ------ Description information type；1---Descriptor_MarkerSet；2---Descriptor_RigidBody；3---Descriptor_Skeleton；4---Descriptor_ForcePlate

dataDescription.Data.MarkerSetDescription->szName--- Markerset name in description information

dataDescription.Data.MarkerSetDescription->nMarkers--- The number of points contained in the Marketset in the description information

dataDescription.Data.MarkerSetDescription->szMarkerNames[markerIndex]--- The name of the markerIndex-th point contained in the Markerset in the description information

dataDescription.Data.RigidBodyDescription->szName --- Rigid body name in description information

dataDescription.Data.SkeletonDescription->szName --- Skeleton name in description information

dataDescription.Data.SkeletonDescription->nRigidBodies --- The number of bone segments contained in the skeleton in the description information

dataDescription.Data.SkeletonDescription->RigidBodies[boneIndex] .ID;

-- The ID of the boneIndex-th bone segment contained in the skeleton in the description information

dataDescription.Data.SkeletonDescription->RigidBodies[boneIndex] .szName---The name of the boneIndexth bone segment contained in the skeleton in the description information

dataDescription.Data.ForcePlateDescription->nChannels--- The number of channels of the force plate in the description information

dataDescription.Data.ForcePlateDescription->szChannelNames[channelIdx]--- The name of the channelIdxth channel of the force plate in the description information

theClient->FreeDataDescriptions(ps);// Note：need to release ps

ps = nullptr;

（3）、Get data information (note: need to be used correspondingly with (5))

sFrameOfMocapData* data = theClient->GetLastFrameOfMocapData();

data information analysis：

data->iFrame------ Data frame number

data->Timecode------ The time code information of the data requires decoded with(4)

data->TimecodeSubframe------ The time code information of the data requires to be decoded with (4)

data->iTimeStamp------ Data timestamp information

data->nMarkerSets--- Number of Markersets of data

sMarkerSetData* markerset = &data->MocapData[i];

markerset->szName--- The name of the i-th Marketset of the data

markerset->nMarkers--- The number of Markers contained in the i-th Markerset of the data

markerset->Markers[i_Marker][0]--- The X value of the i_Marker point contained in the i-th Markerset of the data

markerset->Markers[i_Marker][1]--- The Y value of the i_Marker point contained in the i-th Markerset of the data

markerset->Markers[i_Marker][2]--- The Z value of the i_Marker point contained in the i-th Markerset of the data

data->nRigidBodies--- The number of rigid bodies in the data

data->RigidBodies[i].ID--- The ID of the i-th rigid body of the data

data->RigidBodies[i].x--- The X value of the i-th rigid body of the data

data->RigidBodies[i].y--- The Y value of the i-th rigid body of the data

data->RigidBodies[i].z--- The Z value of the i-th rigid body of the data

data->RigidBodies[i].qx--- The rotation information quaternion Qx of the i-th rigid body of the data

data->RigidBodies[i].qy--- The rotation information quaternion Qy of the i-th rigid body of the data

data->RigidBodies[i].qz--- The rotation information quaternion Qz of the i-th rigid body of the data

data->RigidBodies[i].qw--- The rotation information quaternion Qw of the i-th rigid body of the data

data->RigidBodies[i].nMarkers--- The number of Markers contained in the i-th rigid body of the data

data->RigidBodies[i].MarkerIDs[iMarker]--- The ID of the iMarker-th Marker contained in the i-th rigid body of the data data->RigidBodies[i].Markers[iMarker][0]--- The X value of the iMarker-th Marker contained in the i-th rigid body of the data

data->RigidBodies[i].Markers[iMarker][1]--- The Y value of the iMarker-th Marker contained in the i-th rigid body of the data

data->RigidBodies[i].Markers[iMarker][2]--- The Z value of the iMarker-th Marker contained in the i-th rigid body of the data

data->nSkeletons--- The number of data skeletons

data->Skeletons[i].skeletonID--- The ID of the i-th skeleton of the data

data->Skeletons[i].nRigidBodies--- The number of bone segments contained in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].ID--- The ID of the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].x--- The X value of the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].y--- The Y value of the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].z--- The Z value of the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].qx--- The rotation information quaternion Qx of the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].qy--- The rotation information quaternion Qy of the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].qz--- The rotation information quaternion Qz of the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].qw--- The rotation information quaternion Qw of the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].nMarkers--- The number of Markers contained in the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].MarkerIDs[iMarker]--- The ID of the i-th Marker contained in the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].Markers[iMarker][0]--- The X value of the i-th Marker contained in the j-th bone segment in the i-th skeleton of the data

data->Skeletons[i].RigidBodyData[j].Markers[iMarker][1]--- The Y value of the i-th Marker contained in the j-th bone segment in the i-th skeleton of the data data->Skeletons[i].RigidBodyData[j].Markers[iMarker][2]--- The Z value of the i-th Marker contained in the j-th bone segment in the i-th skeleton of the data

data->nOtherMarkers--- The number of unnamed markers in the data

data->OtherMarkers[i][0]--- The X value of the iMarker unnamed Marker of the data

data->OtherMarkers[i][1]--- The Y value of the iMarker unnamed Marker of the data

data->OtherMarkers[i][2]--- The Z value of the iMarker unnamed Marker of the data

data->nAnalogdatas--- Number of analog channels of data

data->Analogdata[i]--- The data of the i-th analog channel of the data

data->nLabeledMarkers--- The number of all named points of the data

data->LabeledMarkers[iLabeledMarkers].x--- The X value of the iLabeledMarkers-th named point of the data

data->LabeledMarkers[iLabeledMarkers].y--- The Y value of the iLabeledMarkers-th named point of the data

data->LabeledMarkers[iLabeledMarkers].z--- The Z value of the iLabeledMarkers-th named point of the data

data->LabeledMarkers[iLabeledMarkers].size--- The size of the iLabeledMarkers-th named point of the data

（4）、Time information decoding

int hour, minute, second, frame, subframe;

bool bValid = theClient->DecodeTimecode(data->Timecode, data->TimecodeSubframe, &hour, &minute, &second, &frame, &subframe);

（5）、Release data information memory (note: To be used in correspondence with (3))

theClient->NokovFreeFrame(data);

（6）、The code added at the end to disconnect from the motion capture system

theClient->Uninitialize();

- Data description

（1）、The coordinate system is a right-handed system

（2）、For Markers that have been defined in the Markerset, if points are lost or cannot be recognized due to software operation problems, Marker occlusion, etc., the X, Y, and Z coordinate values will be filled in as 9999999.000000