XingYing_SDK is a communication plug-in written in C++ language, in the form of a dynamic library DLL plug-in. By including the SDK into the C++ project and calling the SDK's public interface, Makrer point,rigid body information and force plate including other simulation channel information can be obtained. Use reference SampleClient.

- Establish connection and communication:

- Create a NokovSDKClient object and enter the IP address of the XingYing target. If running on the same computer as NokovSDKClient, enter "10.1.1.198". The C++ code looks like this:

// The first parameter is the SDK broadcast IP of XINGYING software,see:Setting =>NetWork =>LocalAddress

// ServerIP:10.1.1.198

NokovSDKClient* theClient = new NokovSDKClient();

theClient->Initialize(“10.1.1.198”);

- Received data:

  - Set the callback function to receive the transferred data. The C++ code is as follows:

void DataHandler(sFrameOfMocapData* data, void* pUserData);

theClient->SetDataCallback(DataHandler, theClient);

- Data parsing (reading Marker data and rigid-body data) :

- Get all MarkerSet number:

int markerSetNumber = data.nMarkerSets;

- Obtain the position information of the j-th marker of the i-th MarkerSet:

float point = data.MocapData[i].Markers[j][0]; // 0-x 1-y 2-z

- Get all rigid bodies number:

int number = data.nRigidBodies;

- Get the attitude information of the i-th rigidbody(position,rotation) :

float point = data.RigidBodies[i].x; // x y z qx qz qw

- Get all NamedMarker number:

int number = data.nLabeledMarkers;

- Obtain the location information of the i-th NamedMarker:

float point = data.LabeledMarkers[i].x; // x y z

- Get all UnNamedMarker number:

int number = data.nOtherMarkers;

- Obtain the location information of the i-th UnNamedMarker:

float point = data.OtherMarkers[i][0]; //0-x 1-y 2-z

- Get the number of analog channels:

int number = data.nAnalogdatas;

- Get the data of the i-th analog channel:

float point = data.Analogdata[i];

- Get the absolute timestamp of the frame:

long long timestamp = data.iTimeStamp //Number of milliseconds since 1970-01-01

- Use of force plate:

  - Initialize the force plate:

// Wait for the force plate to be initialized,0ms indicates infinite wait.

retCode = theClient->WaitForForcePlateInit(0);

  - Set the callback of force plate:

void ForcePlateHandler(sForcePlates* pForcePlate, void* pUserData);

theClient->SetForcePlateCallback(ForcePlateHandler, theClient);

  - Analyzing the data of the force plate, the force (N) on the X-axis of the i-th force table is as follows:

pForcePlate->ForcePlates[i].Fxyz[0];

- Close the connection:

theClient->Uninitialize();